SPECIFICATIONS

A DIFFUSED AIR AERATION SYSTEM

DEEP WATER APPLICATIONS

1.0	GENERAL

1.1	DESCRIPTION

- Manufacturer shall furnish a diffused air aeration system capable of pumping a synergistic air/water lift from the bottom of a body of water to maximum depth of 35’.

- An on-shore compressor, powered by an electric motor, shall pump air through an individual air supply tube to a bottom-mounted diffuser assembly located in the body of water.

- The air shall be dispersed through a diffuser assembly that is then transferred vertically with entrained water to the surface of the water body, utilizing micro-bubble technology.

- This continuous action shall effectively mix, de-stratify, and transfer atmospheric oxygen throughout the body of water.

1.2	AERATOR COMPONENTS DESCRIPTION

- Compressor – 1/2HP dual head, 1 phase, wobble piston built for continuous operation and equipped with thermal overload protection, an integrated check valve and pressure relief capabilities. Unit shall be oil-free and require no scheduled maintenance other than periodic replacement of the air filter. Compressor shall produce 4.2CFM @ 25PSI and operate at approximately 5.0 running amps @ 120 volts, 2.5 running amps @ 240 volts.

- Compressor Cabinet – Enclosure shall be cylindrical in design and be constructed of minimum 16-gauge stainless steel. Enclosure shall be fully gasketed and equipped with padlock for security, ventilation to provide forced air circulation and an integral cooling fan with thermal protection, producing 115 CFM to guard against excessive compressor operating temperatures.

Diffuser Assemblies – I. Single Membrane – consists of (1) 12" diameter, self-cleaning, flexible, fluoroelastomer layered, fine bubble, EPDM membrane. It shall provide superior resistance to fouling, calcium scaling, chemicals, fats, oils, grease, hydrocarbons, fuels and solvents.

II. Dual Membrane – consist of (2) self-cleaning, 9” diameter, flexible membrane diffusers of EPDM compound with 100% rebound memory, each producing millions of fine bubbles.

- Weighted Air Supply Tubing – Self-weighted, direct burial submersible tubing for connection from compressor to diffuser assembly.  Tubing shall be of flexible PVC composite construction for use with standard PVC solvent weld cement and 0.5” insert fittings. 1/2” tubing shall have 0.52” ID X 1.06” OD, 5/8” tubing shall have .63” ID X 1.15” OD and both have a .27” wall thickness, for long term durability and protection against punctures.  It shall remain flexible in cold temperatures.

- DIFFUSED AIR AERATION DETAIL SPECIFICATIONS

  - DETAILED INFORMATION

  - This specification is intended to provide prospective bidders the necessary information pertaining to the diffused air aeration system specified for the __________________________________________________________ Project.

  - The compressor shall be 1/2HP dual head, operating at _______ Volts (120 or 240), 60 Hertz, 1 Phase.

  - The MODEL specified shall be AquaAir® 2 MODEL NUMBER 9420 (120V) OR 9422 (240V) (circle choice). It shall come complete with cabinet, compressor, Single Membrane or Dual Membrane diffusers (circle one) and   __      feet of weighted air supply tubing. Tubing lengths are as follows: Diffuser A _____ feet, Diffuser B ____ feet.

  - MODEL NUMBER specified in 2.3 shall be base mounted. Base color must be specified; Black-BK, Chocolate-CH, Gray-KG, Tan-TA or Olive-OL (circle one).

3.0	INSTALLATION

The cabinet and compressor must be installed in accordance with the installation instructions, in compliance with all local and National Electrical Code requirements.  This should be done by a licensed electrical contractor.  Any alterations to or substitution for items in this system, unless allowed by the installation instructions, will void the manufacturer’s warranty.  It may also create a hazardous installation.  Read the instructions thoroughly before starting the installation and follow them carefully throughout.

4.0	SAFETY TESTING

The diffused air aeration system shall be tested and approved as a complete unit.  This approval must meet Underwriters Laboratories Inc. requirements in compliance with Category 1450: Motor Operated Air Compressors.

5.0 	ACCEPTABLE MANUFACTURER

This diffused air aeration system, as specified in Section 2, shall be manufactured by AQUAMASTER® FOUNTAINS AND AERATORS, 16024 CTH X, Kiel, WI  53042, (800) 693-3144, or approved equal.

6.0	WARRANTY

All AquaMaster® AquaAir® Diffused Air Aeration Systems are covered under warranty at 100% replacement costs should it fail due to defects in materials or workmanship for a period of 5 years, with the exception of the compressor and cooling fan, which will be  covered under warranty for a period of 2 years. Warranty is in effect from the date of shipment, when given normal and proper usage as determined by the seller upon examination, and when owned by the original user.